| 1. Weather & Site Safety - Chance of precipitation? - Wind speed under 15 knots (20 mph) - Kp-Index is 3 or less - Cloud base is at least 500 feet - Visibility is at least 3 statute miles (SM) - Anti-collision lights needed? - Establish take-off, landing and emergency hover zones - Potential for electromagnetic interference? - Look for towers, wires, buildings, trees and other obstructions - Look for pedestrians and animals and set up safety perimeter, if needed - Discuss flight mission with other crew members, if present 3. Power Up - Turn on RC and open up flight app - Turn on aircraft - Verify connection b/w RC and aircraft - Position RC antennas toward the sky - Verify display panel / FPV screen is functioning properly - Calibrate Inertial Measurement Unit (IMU) as needed - Calibrate compass as needed - Verify battery levels on both transmitter and aircraft - Verify connection with at least six satellites for GPS and that I hear "The Home Point has been updated" prior to taking off (this is specific to the DJI Fly app) | 2. System Inspection - Registration number is displayed properly and is legible - Look for abnormalities — aircraft frame, propellers, motors, undercarriage - Look for abnormalities — gimbal, camera, transmitter, payloads, etc. - Gimbal clamp and lens caps are removed - Clean lens with microfiber cloth - Attach/insert propellers, battery/fuel source, SD card, lens filters 4. Take Off - Take off and hover at an altitude of 10 feet for 10 seconds - Look for any imbalances or irregularities - Listen for abnormal sounds - Pitch, roll, and yaw to test control response and sensitivity - Check for electromagnetic interference or other software warnings - Do one final check to secure safety of flight operations area - Proceed with flight |
| --- | --- |